Mobile Security Project: IMO App Analysis Report
Environment: Windows 11, Android Studio Emulator (Google APIs 35 image)
Target App: IMO (com.imo.android.imoim)

1. Objective
The primary goal of this forensic analysis is to examine the IMO Android application (com.imo.android.imoim) installed on an emulator. The analysis focuses on:

- Identifying key data storage locations (databases, preferences, and files).

- Quantifying the size and number of files in sensitive directories.

- Assessing the application's data storage practices for security and privacy implications.

- Utilizing standard mobile forensics tools (ADB, Andriller) for data extraction and parsing.

2. Tools Used

| Tool | Purpose |
| --- | --- |
| Android Studio Emulator | Simulated Android device for controlled testing. |
| ADB (Android Debug Bridge) | Device communication and file extraction (adb pull) from the rooted emulator. |
| Andriller | Automated forensic parsing and decoding of app data (e.g., SQLite databases, Shared Preferences). |
| Python 3.x | Script execution environment for report generation and custom analysis. |
| DB Browser for SQLite (DB4S) | Manual inspection of extracted SQLite databases. |

3. Environment Setup & Configuration

- Android Studio Emulator initialized with a rooted AOSP image.

- ADB connectivity verified using adb devices.

- Local app data extraction path: C:\Users\Brygh\Desktop\Mobil Security Project\imo_appdata

- External storage extraction path: C:\Users\Brygh\Desktop\Mobil Security Project\imo_external

- Required Python libraries installed (python-docx, andriller).

4. Data Extraction

4.1 Attempted ADB Backup

adb backup -apk -obb -shared -f imo_backup.ab com.imo.android.imoim

Outcome: A backup file was created (typically 1 KB), but it was largely empty, demonstrating the general unreliability of adb backup on modern Android versions (12+) or for apps with encrypted data.

Using Andriller to extract the .ab file and DB Browser for SQLite to inspect internal databases, it was possible to access the messages I sent and the initial call logs, showing that some data could still be recovered despite the limitations of the backup.

4.2 ADB Pull of App Data (Rooted Emulator)
Internal app data was successfully pulled from /data/data/com.imo.android.imoim/ to the local path:
C:\Users\Brygh\Desktop\Mobil Security Project\imo_appdata

Extracted Data Summary (placeholders):

| Folder | File Count | Size (MB) |
| --- | --- | --- |
| Databases | 5 | 1.25 |
| Shared Preferences | 12 | 0.04 |
| Files | 45 | 5.80 |
| Cache | 150 | 12.55 |
| External Storage | 0 | 0.00 |

5. Analysis & Observations

5.1 Analysis with Andriller

python -m andriller --decode "C:\Users\Brygh\Desktop\Mobil Security Project\imo_appdata" --output "Parentfolder"

- Andriller successfully parsed SQLite database schemas and shared preferences.

- Message content in main chat databases (e.g., imo_friends_...) was found encrypted (BLOB data), preventing direct viewing without the corresponding decryption key.

- Indicates strong application-level encryption protecting sensitive user data.

5.2 Key Security Findings

- Strong Encryption: IMO employs strong local encryption for messages, requiring a specific key (likely stored in Android Keystore or a protected file) for content recovery.

- Root Access Requirement: Full extraction of sensitive data (databases, preferences) was only possible with root access, highlighting Android’s standard permission protections.

- Sensitive Paths: The shared_prefs directory contains crucial configuration files, tokens, or encryption metadata requiring further binary analysis.

7. Conclusion
The analysis identified the location and size of sensitive IMO application artifacts. The primary finding is the presence of strong application-level encryption securing the chat database content. Recovery of plaintext messages requires runtime key extraction or reverse engineering.